Primary-Backup Scheduling in MultiCore Real-Time Embedded Systems

Research Area:

MultiCore for Real-Time Embedded Applications

Authors:

A. K. Kamesh

Kartik S

Namratha M S

Anusha K

Faculty mentor:

Mr. D Krupesha

Name of the Institution:

PES Institute of Technology

Abstract:

Fault Tolerance is crucial in Hard Real-Time Embedded Systems. The Primary-Backup (PB) technique is a popular method used for achieving fault tolerance in such systems. This technique is used to ensure that all tasks complete within their deadlines in the event of a processor failure.

Multicore Processors are now being specially designed for use in Embedded Systems. One consequence of using a multicore processor, instead of equivalent number of uniprocessors, is that when there is a serious processor fault, all the cores in that processor can fail simultaneously.

In this paper, we have adapted the Primary-Backup technique for use in a Multicore-Multiprocessor Real-Time Embedded System, to handle the situation where multiple cores of a processor can fail at the same time. We have assumed that the tasks are aperiodic and non-preemptive.

When a processor fault occurs, the backups of tasks running on all the cores of the faulty processor may have to be executed. This increases redundancy and hence decreases the effective CPU utilization, especially when the system is recovering from the fault. We have proposed a heuristic for reducing this problem.

Background:

Embedded Systems typically have real-time constraints. A real-time system not only has to produce correct results, but also produce them within a strict deadline.

There are two types of real-time constraints – soft constraints and hard constraints. A system is said to have hard real-time constraints if failure to meet the deadline has catastrophic effects. Some examples are – Patient monitoring systems, Missile, Nuclear-plant and Rocket control systems. If missing the deadline does not have disastrous effects, then the system is said to have soft real-time constraints. Some examples are airline and train reservation systems.

In recent years, embedded systems have used parallel processing to achieve higher performance and throughput. Such systems have used multiple processors to perform different tasks simultaneously. However, the use of multiple processors is restricted by the power consumption and cost involved.

Multicore processors are now being adapted for use in embedded systems. These processors are now specially manufactured for low power consumption, while still providing the increase in performance required. Embedded systems benefit from using multicore processors, instead of equivalent number of uniprocessors, in the following ways –

- Good performance per Watt of Power

- Cost Effectiveness

- Lower Power Consumption

- Lesser amount of time required for Cache Snooping

- Simplification of Integrated Circuit Boards

- Increased Scalability and Flexibility

The main reason for the focus on multicore processors is that it is not possible to indiscriminately increase the clock speed of a uniprocessor in order to increase performance. Moreover, a higher clock speed leads to higher power consumption. The solution is parallelism using multicore processors.

A critical requirement of a real-time embedded system using multiple processors is that it should be Fault Tolerant. Fault Tolerance is the ability of the system to produce timely and correct results even when faults occur.

The Primary-Backup (PB) technique is widely used to provide fault tolerance in multiprocessor systems. In this technique, a task (called the primary) is run in one of the processors. If that processor fails for some reason, and the task is not completed, then a copy of that task (called the backup) is executed in another processor, such that the task still executes within the deadline. The PB method requires that the scheduler schedules both the primary as well as the backup before actually executing the task. If the task cannot be scheduled, it is rejected. No task is scheduled unless it is certain that it will complete within the deadline, even if one of the processor fails.

We have assumed all tasks to be aperiodic and non-preemptive. A task is said to be aperiodic if it gets activated when an asynchronous event occurs. A non-preemptive task cannot be interrupted while it is being executed.

Each task can be represented by the tuple –

< arrival time , deadline , worst-case computation time >

Consider a system with 3 processors P1, P2 and P3, and Tasks T1, T2 and T3 scheduled as shown below –

T1 < 0 , 25 , 10 > ; T2 < 0 , 20 , 7 > ; T3 < 0 , 30 , 5 >

5

10

15

20

25

30

T1

T3

P1

T2

T1

P2

T2

T3

P3

Time

Backup

Primary

If the processor P1 fails while executing T1, then the backup of T1 is executed on processor P2.

From the example above, it is clear that the following conditions should hold for all tasks –

- The time slots of the Primary and Backup should not overlap.

- ( Deadline – Arrival Time ) >= 2 * Worst-case Computation Time

This is because there should be sufficient time for both the primary and backup to be executed, if needed.

If failures occur rarely, then the backup slots remain unutilized. In order to solve this problem, the backup slot of a task T is freed, if possible, as soon as the primary of T is completed successfully. This increases CPU utilization. Another method is to overload time slots on the same processor with more than one backup. This technique is called Backup-Backup overloading. Since the backup slots can be shared, the number of backup slots decreases. However, the following condition must hold –

If the backups of two tasks T1 and T2 overlap on the same processor, then the primaries of T1 and T2 cannot be scheduled on the same processor.

This is logical since, if the primaries of T1 and T2 were executed on the same processor, and that processor fails, then only one of the tasks can be re-executed in the backup slot. One more aspect of this method is that during the time in which the system is recovering from the processor fault by executing backups, another processor cannot fail.

The PB method has been successfully implemented in hard-real time embedded systems. However, it does not consider the situation where more than one processor fails simultaneously. In a multicore-multiprocessor system, if a processor fault occurs, then in the worst case all of the cores of that processor fail.

We have tried to adapt the PB method so that it will work even in this scenario. In addition, we have proposed a heuristic in order to reduce the task redundancy when the system is recovering from a failure.

More information about the PB method can be found in [1].

Problem Statement:

Consider a Real-Time Embedded System consisting of N processors P1, P2, P3, …, PN, where each of these processors may possibly have multiple cores.

The system is driven by asynchronous events, and hence runs aperiodic tasks, which are assumed to be non-preemptive. Each of these tasks is denoted by the tuple –

< arrival time, deadline, worst-case computation time >

such that the window of the task (deadline – arrival time) is at least twice as large as the worst-case computation time.

Every task T has to be scheduled on the system in such a way that it will still complete within the deadline even in the event of a processor fault, during which all cores of that processor might fail simultaneously. In addition, T has to be scheduled such that the amount of repetition of tasks when a fault occurs is reduced.

Methodology:

In a system consisting of multiple uniprocessors, the PB method does not work if there is more than one processor fault simultaneously. Similarly, in a multicore system, the PB method may not work if all the cores of a processor fail at the same time. This is illustrated by the following example –

Consider a system consisting of 2 processors, each having multiple cores as shown –

T1

P1

Core1

T2

Core2

T2

Fault

Core3

Core4

P2

T3

T1

Core1

T3

Core2

5

10

15

20

25

30

Time

Backup

Primary

T2 has its primary scheduled on Core2 of Processor P1, and the Backup scheduled on Core3 of the same processor. Similarly, T3 has its primary scheduled on Core1 of Processor P2, and the backup scheduled on Core2 of the same processor. Clearly, when Processor P1 fails because of a fault (and all its 4 cores fail), as shown in the figure, then tasks T1 and T2 are incomplete. It is possible to run the backup of T1 on Core1 of the other processor P2, which is still operating. However, we cannot run the backup of task T2 since the Core3 of processor P1 has failed. A similar situation occurs if processor P2 fails – then T3 can no longer meet the deadline.

From the above example, it is clear that if the primary of a Task T is scheduled on processor P1, then the Backup of T has to be scheduled on a different processor; it is not sufficient to schedule it on a different core of the same processor to achieve fault tolerance.

Hence, the PB method has to modified slightly such that the following condition holds while scheduling backups –

[C1] If the Primary of a Task T is scheduled on a core of a Processor Pi, and the Backup of T is scheduled on a core of a Processor Pj, then Pi != Pj

Since there is now a restriction on the cores on which the backups can be scheduled, there would be a decrease in CPU utilization. However, we have considered the worst-case scenario where all the cores of the processor fail. For less serious faults, this restriction can be relaxed. (For example, if only 2 cores of the 4 core processor can fail simultaneously)

One possible way of scheduling the backups in the above example so that the stated condition holds is shown below –

Fault

5

10

15

20

25

30

Time

P1

Core1

Core2

Core3

Core4

Core1

Core2

P2

T1

T2

T3

T1

T2

T3

Backup

Primary

Fault

5

10

15

20

25

30

Time

P1

Core1

Core2

Core3

Core4

Core1

Core2

P2

T1

T2

T3

T1

T2

T3

Backup

Primary

T4

T4

Consider the same embedded system with different tasks as shown below –

A fault occurs on Processor P1 at the time shown. As a result, the backup of tasks T1 and T2 are executed. However, it can be seen that since the fault occurred towards the end of tasks T1 and T2, a large amount of the code of T1 and T2 has to be re-executed, increasing redundancy. The situation worsens if the number of cores on the faulty processor is more, with each core assigned a task.

We made the following observation –

In a processor with a large number of cores, if relatively longer tasks are assigned to each core, then the amount of redundancy increases when a fault occurs on that processor.

In the above example, P1 is the processor with the larger number of cores (four). It has been assigned tasks T1 and T2. These two tasks are relatively longer compared to tasks T3 and T4. When the fault occurred, the amount of code to be re-executed was large.

However, if the smaller tasks T3 and T4 had been assigned to the processor P1 as shown, then the redundancy decreases –

P1

Core1

T3

T1

T2

T4

Core2

Fault

Core3

Core4

P2

T1

T3

Core1

T2

T4

Core2

5

10

15

20

25

30

Time

Backup

Primary

From this observation, we concluded that if smaller tasks are assigned to the processor with larger number of cores, then the redundancy decreases.

Hence, we have proposed the following heuristic while scheduling primaries –

Tasks of relatively smaller length should be preferably scheduled on the processor with the more number of cores. Longer tasks should be scheduled on the processor with smaller number of cores.

A more detailed discussion of the above heuristic is given below.

Consider an embedded system with N processors P1, P2, …, PN. Processor P1 has N1 cores, processor P2 has N2 cores, and so on, such that N1 >= N2, N2 >= N3 … i.e. the processors are numbered in the decreasing order of the number of cores.

Let the minimum length of the tasks to be executed on that system be “min”, and the maximum length be “max”. These two parameters can be estimated before the embedded system is deployed.

Divide the range [min, max] into N parts W1, W2, …, Wn as shown. Assign the processors to each of these parts as shown –

P1

P2

P3

P4

Pn

…

Min

Max

W1

W2

W3

W4

Wn

Consider a Task T of length L to be scheduled.

We can easily find out in which part (Wi) of the range [min, max] the value of L lies. When the primary for task T is being scheduled, then the scheduler should use the following heuristic –

- If the task T belongs to the part Wi of the range [min, max], then try scheduling the task on the cores of processor Pi

- If it cannot be scheduled on Pi, then try scheduling it on the cores of processor P(i+1)

- Similarly, if it is not possible on P(i+1), then try scheduling on P(i+2), and so on, till Pn

- If it not possible to schedule it on any of the processors following Pi, then try the remaining processors in this order – P(i-1), P(i-2), …. and so on till P1

- If it is not possible to schedule the task on any of the processors, then it is rejected

The PB method is independent of the actual method used to schedule the primaries and backups on the processors. Similarly, for the above heuristic, for each processor, any algorithm can be used to schedule the primary on the cores.

The backup of the task T can also be scheduled using any algorithm, as long as the earlier stated condition C1 holds.

Consider a hypothetical embedded system with 5 processors, with P1 having 8 cores, P2 having 4 cores, P3 having 4 cores, P4 having 2 cores, and P5 having only 1 core. Suppose the maximum length of a task is 200 and the minimum length of a task is 10. Then, min = 10, max = 200. Dividing the range [10, 200] into 5 equal parts, we find each part = (200 – 10) / 5 = 38 time units.

P1

P2

P3

P4

P5

10

48

124

86

200

W1

W2

W3

W4

W5

162

Consider a task T < 20 , 120 , 100 >. Its length is 100. It belongs to the part W3 of the range (min, max). Hence, we should try scheduling it on the processors in the following order –

P3, P4, P5, P2, P1

This heuristic ensures that tasks are distributed on the processors based on the length of the tasks.

An additional improvement can be deduced from the above example using the observation that P2 and P3 have the same number of cores (four). If both P2 and P3 have equal probability of failing, then the parts W2 and W3 can be joined into one part W. Hence, the task T above can now be scheduled on either P2 or P3 initially, instead of giving preference to P3.

The above heuristic does not consider how the task lengths are distributed over the range [min, max]. In particular, most of the task lengths may be clustered in one of the parts Wi. We now try to generalize the heuristic so that it takes into consideration the actual distribution of task lengths.

Consider a set of possible tasks on an embedded system such that their lengths are normally distributed over the range [min, max]. In practice, a normal distribution may not be suitable, but the basic idea remains the same irrespective of the type of distribution. A normal distribution of the task lengths is shown below –

Number

of Tasks

Pi: Processor i

Ai: Sub Region i

under the

Distribution

Curve

Wi: Part i of the

Range [min, max]

P2

P3

P1

P4

P5

A1

A2

A3

A4

A5

W1

W2

W3

W4

W5

Min

Max

Task

Length

The curve shows the distribution of the task lengths. Consider the area bounded by this curve and the x-axis. We can divide this area into N sub-regions A1, A2, A3, …, An (N = 5 in the figure), and allot a processor to each sub-region as shown. The region under the curve is divided such that the ratio of the areas of the sub-regions is proportional to the number of cores in the processors, i.e.

A1 : A2 : A3 : A4 : A5 = N1 : N2 : N3 : N4 : N5

By doing this, processor P1, which has 8 cores (see the preceding example), will be allotted the smaller tasks, as usual. In addition, since processor P1 is capable of running a larger share of the tasks, as compared to the processors with lesser number of cores, the area A1 is also proportionally larger. Hence, processor P1 will be allotted larger number of smaller tasks, while processor P5 will be allotted smaller number of larger tasks. This logic can be applied to any type of distribution curves.

The calculations above can be performed before the embedded system is actually deployed. The scheduler does not have to perform any extra calculations. The parts W1, W2, W3, …, Wn of the range [min, max] will differ based on the distribution of task lengths.

The smaller tasks need not be scheduled solely based on the number of cores. Instead, smaller tasks can be scheduled on processors which not only have larger number of cores, but also have higher probability of failing. This ensures that the amount of redundancy is reduced during a processor fault.

Scope for future work:

All the tasks have been assumed to be independent of each other and having the same priority. Moreover, all tasks have been assumed to be aperiodic. However, an embedded system typically runs periodic tasks as well. The PB method can be modified to consider the presence of periodic tasks being scheduled on the system. Furthermore, research can be done to find out a suitable heuristic to schedule interdependent tasks, with priority.

Conclusion:

In this paper, we adapted the Primary-Backup Technique to provide Fault Tolerance to a system using Multiple Multicore Processors. We modified the method of scheduling the backups so that the system still operates correctly even when all the cores of a processor fail.

Next, we attempted to reduce the amount of redundancy while the system is recovering from the processor fault. We proposed a heuristic that preferably allotted tasks of smaller length to the processors with more number of cores. This would reduce the amount of code that would have to be re-executed when a fault occurs.

Then, we generalized the heuristic to consider the actual distribution of task lengths, and assigned the processor with more number of cores a larger share of the smaller tasks.

Finally, we concluded that additional factors could govern the scheduling of smaller length tasks: for example, the probability of a particular processor failing.

References:

[1] Sunondo Ghosh, Rami Melhem, and Daniel Mossé, “Fault-Tolerance Through Scheduling of Aperiodic Tasks in Hard Real-Time Multiprocessor Systems”, IEEE Transactions on Parallel and Distributed Systems, Vol. 8, No. 3, March 1997, pg. 272 - 284

Acknowledgements:

We would like to express our heartfelt thanks to Mr. D Krupesha, our mentor. He was the person who encouraged us to participate in this competition. He is an avid reader of research papers, and provided us with very useful pointers and tips to write a good research paper.

Finally, we would like to thank our families for their encouragement and support.